# KENYA CERTIFICATE OF BASIC EDUCATION

## SENIOR SCHOOL ASSESSMENT

### AUGUST ASSIGNMENT 2026

GRADE 10 – POWER MECHANICS
Time: 2 Hours 30 Minutes

LEARNER’S DETAILS

Name: _____________________________________________________________.  School: ____________________________________________.

Assessment Number: _______________________________________________.      Date: ____________________________________________.

School Code: ______________________________________________________.        Signature: _______________________________________.

### INSTRUCTIONS TO CANDIDATES

- Write your name in the spaces provided above.

- Write the name of your school and your stream in the spaces provided.

- Write your admission number and the date of the assessment in the spaces provided.

- This paper consists of two sections: A and B.

- Answer all questions in section A and section B.

- Answer the questions in English.

- All answers MUST be written in the spaces provided in the paper.

- Do NOT remove any page from this question paper.

- For technical drawing questions, ensure all construction steps are clearly shown

### FOR OFFICIAL USE ONLY

| SECTION | SECTION A | SECTION B | % SCORE | EE1 | EE2 | ME1 | ME2 | AE1 | AE2 | BE1 | BE2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCORE RANGE | 30 MARKS | 50 MARKS |  | 90-100 | 75-89 | 58-74 | 41-57 | 31-40 | 21-30 | 11-20 | 1-10 |
| POINTS |  |  |  | 8 POINTS | 7 POINTS | 6 POINTS | 5 POINTS | 4 POINTS | 3 POINTS | 2 POINTS | 1 POINT |
| LEARNER’S TOTAL SCORE |  |  |  |  |  |  |  |  |  |  |  |

## SECTION A (30 MARKS)

Answer ALL questions in this section.

- Three learners at Muranga School are discussing their future in the Power Mechanics industry.

Kamau: "I want to design new engine systems for electric cars."

Mutua: "I want to work in the Service and Repair sector at a garage in Machakos."

Otieno: "I am interested in Marine Engineering, working on ship engines in Mombasa."

(a) Identify the specific career path Kamau is interested in. (1 mark)

__________________________________________________________

(b) Name the branch of power mechanics that Otieno’s interest falls under. (1 mark)

__________________________________________________________

(c) List three activities related to power mechanics that can be found in a local shopping center. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

- Study the workshop diagram below showing a standard power mechanics layout.

A								D

(a) Identify the area labeled A and D. (2 marks)

A: ____________________________

D: ____________________________

(b) Select the correct Personal Protective Equipment (PPE) from the list below that a learner must wear when handling a battery in the workshop:

Respirator           Rubber Gloves         Welding Shield                                 (1 mark)

(c) Explain the importance of maintaining clear Passageways (D) in a workshop. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- A Grade 10 learner at Amapera Senior School is inspecting a tire with the following markings: 205/55 R 16.

(a) Identify what the letter R stands for in the specification. (1 mark)

__________________________________________________________

(b) Distinguish between Tube and Tubeless tires based on their construction. (2 marks)

| Tube tire | Tubeless tire |
| --- | --- |

(c) Name the two types of tire ply orientations used in vehicles. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- Technical drawing is essential for power mechanics. Study the mechanism below.

(a) Define the term "Locus" as used in technical drawing. (1 mark)

______________________________________________________________________________________________________________________________________________________

(b) Identify the type of locus produced by a point on a circle rolling along a straight line. (1 mark)

______________________________________________________________________________________________________________________________________________________

(c) Outline the method for constructing an Internal Tangent to two unequal circles. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

- (a) Match the vehicle body type in Column A with its correct description in Column B. (5 marks)

| Column A | Column B |
| --- | --- |
| (i) Station Wagon | (A) A vehicle with an open cargo area and a cabin. |
| (ii) Convertible | (B) A car with a roof that can be folded down or removed. |
| (iii) Pickup | (C) A car with an extended roof and a hatch at the back for luggage. |
| (iv) Saloon | (D) A large vehicle designed to carry many passengers. |
| (v) Bus | (E) A standard car with four doors and a separate boot. |

(b) Identify the two types of chassis construction shown in the images below. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

(c) State three functions of a vehicle chassis. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

### SECTION B (50 MARKS)

- Below is an exploded view of a single-cylinder internal combustion engine.

X					Y					Z		W

(a) Name the components labeled X, Y, Z, and W. (4 marks)

X: _________________________       Y: _________________________

Z: _________________________       W: ________________________

(b) Describe the function of the Crankshaft (Y) in relation to the piston’s motion. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(c) Explain the purpose of the Camshaft (W) in the valve timing mechanism. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(d) Compare the material used for the Cylinder Block versus the Sump. (3 marks)

| Cylinder block | Sump |
| --- | --- |

(e) Identify the component that seals the top of the cylinder block. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- A mechanic in Thika is performing a precision measurement on an engine block.

S							T

(a) Identify the tools being used in the pictures above. (2 marks)

S: _________________________________

T: _________________________________

(b) Explain why it is necessary to measure the Cylinder Bore for "ovalness" and "taper." (4 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(c) Describe the procedure for measuring the Piston Ring Gap using a feeler gauge. (4 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(d) Calculate the engine capacity in cm3 if the bore is 80mm and the stroke is 90mm (use π=3.142). (3 marks)

(e) Mention the unit used to express Engine Power in the SI system. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- (a) Distinguish between an Internal Combustion Engine (ICE) and an External Combustion Engine (ECE) using the table below. (4 marks)

| Feature | Internal Combustion | External Combustion |
| --- | --- | --- |
| Location of Combustion |  |  |
| Common Example |  |  |

(b) Write True or False in the following statement:

The "Big Bang" theory is related to the evolution of the first steam engine. [__________] (1 mark)

(c) Identify the engine cylinder layout shown in the diagrams below. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(d) State one key trend in the historical development of automobiles regarding fuel efficiency. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- (a) Identify the three joining components shown below. (3 marks)

i                                                      ii                                                  iii

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(b) Define the terms "Adhesion" and "Cohesion" as used in vehicle body repairs. (2 marks)

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(c) Explain the process of Riveting when joining two metal sheets of a vehicle body. (3 marks)

- ___________________________________________________________________

- ___________________________________________________________________

- ___________________________________________________________________

(d) List two types of threaded fasteners used in engine assembly. (2 marks)

- ___________________________________________________________________

- ___________________________________________________________________

### MARKING SCHEME

LEARNING AREA: POWER MECHANICS GRADE: 10 (SENIOR SCHOOL)

### SECTION A (30 MARKS)

1. Fundamentals & Careers (5 Marks)

(a) Career: Automotive Engineer / Design Engineer. (1mk)

(b) Branch: Marine Power Mechanics / Marine Engineering. (1mk)

(c) Activities: Vehicle servicing (oil change), tire mending/vulcanizing, motorbike (boda boda) repair, welding of exhaust pipes, or battery charging. (Any 3 = 3mks)

2. Workshop Layout and Safety (5 Marks)

(a) Identification: A: Tool room. D: Passageways/Walkways. (2mks)

(b) PPE: Rubber Gloves (Chemical resistant for battery acid). (1mk)

(c) Importance: To ensure free movement of personnel and equipment, and to provide clear exit routes during emergencies (fire or accidents). (2mks)

3. Vehicle Systems: Wheels and Tires (5 Marks)

(a) Identification: R stands for Radial (referring to the ply construction). (1mk)

(b) Distinguish: Tube tires have an inner rubber tube that holds air; Tubeless tires have an airtight inner liner bonded to the casing and seal directly against the wheel rim. (2mks)

(c) Orientations: Radial ply and Cross ply (Bias ply). (2mks)

4. Related Drawing (5 Marks)

(a) Definition: A locus is the path traced by a point as it moves according to a specific mathematical or mechanical rule. (1mk)

(b) Identification: Cycloid. (1mk)

(c) Method:

  - Draw a line connecting the centers of the two circles.

  - Draw a circle with a radius equal to the sum of the two radii (R1​+R2​).

  - Construct a tangent from the center of the smaller circle to this new circle. (3mks)

5. Vehicle Body and Chassis (10 Marks)

(a) Matching: (i)—C, (ii)—B, (iii)—A, (iv)—E, (v)—D. (5mks)

(b) Chassis Types: 1. Separate (Body-on-frame) 2. Integral (Monocoque/Unibody). (2mks)

(c) Functions: To support the weight of vehicle components (engine, body, passengers); to withstand stresses from movement; to provide mounting points for suspension and wheels. (3mks)

### SECTION B (50 MARKS)

6. Engine Components and Functions (15 Marks)

(a) Naming: X: Piston. Y: Crankshaft. Z: Connecting Rod. W: Camshaft. (4mks)

(b) Crankshaft Function: It converts the reciprocating (up and down) motion of the piston into rotary (turning) motion to drive the wheels. (3mks)

(c) Camshaft Function: It controls the opening and closing of the intake and exhaust valves at the correct time during the engine cycle. (3mks)

(d) Comparison: The Cylinder Block is usually made of cast iron or aluminum alloy for strength and heat dissipation; the Sump is often made of pressed steel or light alloy as its main job is to hold oil, not withstand combustion. (3mks)

(e) Component: Cylinder Head. (2mks)

7. Engine Measurements and Tools (15 Marks)

(a) Tools: Inside Micrometer  and Feeler Gauge. (2mks)

(b) Reason: To ensure the cylinder is perfectly round and uniform. "Ovalness" occurs from side-thrust wear; "Taper" occurs because wear is higher at the top (TDC) where heat and pressure are greatest. (4mks)

(c) Procedure: Place the ring into the cylinder, use a piston to push it down so it sits square, and then insert the feeler gauge into the gap to check the clearance. (4mks)

(d) Calculation: Volume=π×r2×h r=40mm=4cm; h=9cm V=3.142×(4×4)×9=452.45cm3. (3mks)

(e) Unit: Watt (W) or Kilowatt (kW). (2mks)

8. Evolution and Classification (10 Marks)

(a) Table:

ICE: Inside the engine (e.g., Petrol/Diesel engine).

ECE: Outside the engine (e.g., Steam engine/Turbine). (4mks)

(b) True/False: FALSE. (1mk)

(c) Layouts: 1. In-line 2. V-engine 3. Opposed (Flat/Boxer). (3mks)

(d) Trend: Movement towards lighter materials, turbocharging, and hybrid/electric systems to reduce fuel consumption and emissions. (2mks)

9. Vehicle Body Joining (10 Marks)

(a) Identification: 1. Bolt & Nut 2. Pop Rivet 3. Circlip. (3mks)

(b) Definitions: Adhesion is the force of attraction between different substances; Cohesion is the force of attraction between the same molecules within a substance. (2mks)

(c) Riveting: A hole is drilled through both sheets, a rivet is inserted, and the "tail" is deformed (expanded) using a rivet gun to lock the sheets together. (3mks)

(d) Fasteners: Studs, Cap screws, or Machine bolts. (2mks)